# EXPERIMENT NO : 4

## STUDY OF LAN CONFIGURATION

### AIM

To establish a Local Area Network (LAN) by interconnecting multiple computers through an Ethernet switch and to verify communication and resource sharing among the connected systems.

### OBJECTIVE

- To understand the basic components of a LAN.

- To configure IPv4 addresses on computers.

- To connect multiple systems using an Ethernet switch.

- To test network connectivity using the ping command.

- To demonstrate file sharing between computers on the same network.

### REQUIREMENTS

- 4 Computers

- 1 Ethernet Switch

- 4 Ethernet/Cat6 Cables

- Windows Operating System

- Command Prompt

### NETWORK TOPOLOGY

A simple star topology is used, where all four computers are connected to a central Ethernet switch.

Example IP Configuration:

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.20.1 | 255.255.255.0 |
| PC2 | 192.168.20.2 | 255.255.255.0 |
| PC3 | 192.168.20.3 | 255.255.255.0 |
| PC4 | 192.168.20.4 | 255.255.255.0 |

### PROCEDURE

- Determine the number of computers and network devices required for the LAN.

- Arrange the computers and switch according to the planned network layout.

- Connect each computer to a separate port on the Ethernet switch using Ethernet cables.

- Check the link indicators on the switch and computers to confirm that the physical connections are active.

- Configure the IPv4 address on each computer:

  - Press Windows + R to open the Run dialog.

  - Enter ncpa.cpl and press Enter.

  - Right-click the Ethernet adapter and select Properties.

  - Select Internet Protocol Version 4 (TCP/IPv4) and click Properties.

  - Choose Use the following IP address.

  - Enter a unique IP address and a common subnet mask for all computers.

- Repeat the IP configuration process for the remaining PCs, ensuring that no two computers have the same IP address.

- If the switch supports web-based management, connect a computer to the switch and open the switch management interface using its configured IP address.

- Log in with the appropriate administrator credentials and verify the switch's network settings.

- Open Command Prompt on one computer.

- Use the following command to test communication with another PC:

ping 192.168.20.2

- Check the responses received from the destination computer.

- Repeat the ping test between different pairs of computers.

- Create a test folder on one computer and enable Network Sharing for that folder.

- From another computer connected to the same LAN, open the network location and access the shared folder.

- Verify that files can be accessed through the LAN.

### OBSERVATION

All four computers are connected to a central switch, forming a star-based LAN. Each system is assigned a unique IP address within the same subnet. Successful ping responses indicate that the computers can communicate with one another through the switch.

The shared-folder test demonstrates that computers connected to the same LAN can exchange resources when appropriate sharing and permission settings are enabled.

### RESULT

Thus, a Local Area Network was successfully configured using multiple computers, Ethernet cables, and a network switch. IP addressing, connectivity testing using the ping command, and file sharing between LAN-connected computers were successfully verified.

### CONCLUSION

The experiment demonstrated the basic working of a LAN and the role of a switch in connecting multiple devices. It also provided practical knowledge of IPv4 configuration, network connectivity testing, and resource sharing within a local network.